Q10 – Additional Access Port

Operator’s Manual

505 Research Drive  Perry, Iowa 50220

Toll-free 800.695.2743  515.465.9363  fax 515.465.9464

www.percival-scientific.com

[This page intentionally left blank]

# Introduction

To help customers create better science, Percival Scientific continuously studies and refines the design of its products in order to provide the best possible solutions and equipment in the industry.  As such, changes in the design and/or operation of equipment may occur without publication.  Please feel free to contact Percival Scientific directly for product updates.

Information contained within this supplemental user manual shall supersede any conflicting information contained within the basic chamber user manual.

Please read this manual carefully before proceeding with operation

# Revision Log

| Version | Date | Description |
| --- | --- | --- |
| Rev A | 6/10/2024 | Initial Release |

Introduction	3

Revision Log	4

Warranty Information	6

Definitions	8

General	8

Size and Location	8

Installation	8

Operation	9

Maintenance	9

Troubleshooting	9

Fit	9

Condensation	10

Parts and Service	10

# Warranty Information

Equipment Limited Warranty

Thank you for purchasing equipment from Percival Scientific, Inc.  Our goal is to provide every customer with an exceptional ownership experience.  We are committed to providing products and services of the highest quality, and your satisfaction is our top priority.  In support of this objective, we present the following warranty.

Warranty Terms

Percival Scientific, Inc. (hereinafter referred to as Percival) warrants that equipment manufactured by Percival shall be free from defect in material or workmanship, when installed, operated and maintained in accordance with instructions provided by Percival.

Warranty Coverage

- Coverage period shall be twenty-four (24) months from the date of delivery

- Percival’s obligation under this warranty is limited to the repair or replacement of defect at no charge for parts, shipping and labor

- Percival reserves the right to inspect the equipment after a warranty claim is made in order to determine the proper course of corrective action

Exclusions

- Parts or elements damaged from improper installation, including mechanical, electrical and plumbing services, IT infrastructure, inadequate room ventilation, improper water quality, and ambient conditions that are out of specification

- Parts or elements damaged from improper operation

- Parts or elements damaged from improper maintenance

- Parts or elements considered to be consumable, that outlive their normal expected terms of usefulness, such as: light bulbs, filters, desiccant material and CO2 scrubbing media

- Damage resulting from shipment to or relocation within the customer’s facility after equipment has been signed for and ownership is transferred to the customer

- Modification to equipment by the customer without Percival’s consent may void this warranty

- Repair work undertaken without prior consent of Percival may void this warranty

- Damage caused by accident, fire, Force Majeure or Acts of God

Additional Terms

- This warranty is in lieu of any other warranties, expressed or implied, including merchantability or fitness for a particular purpose

- Warranty terms described herein cannot be changed without the expressed, written consent from Percival

- The owner agrees that Percival’s sole liability with respect to defective parts or materials shall be set forth in this warranty, and any claims for incidental or consequential damages are expressly excluded

- This warranty is non-transferable and applies only to the original purchaser

- Repaired or replaced parts shall be warranted only for the unexpired portion of the original warranty, unless otherwise defined

- Parts replaced under warranty shall become the property of Percival

To Make a Warranty Claim

- Document the issue with detailed notes and photographs along with the equipment’s serial number

- Contact Percival’s customer support team at customerservice@percival-scientific.com, parts@percival-scientific.com or 1-800-695-2743

- Once your claim is received, our customer support team will review it and contact you with further instructions

# Definitions

# General

Access ports are designed to provide a pathway between a chamber’s interior and its surrounding environment.  These ports are commonly used to pass instrumentation, cabling or tubing into an environment, while allowing its door to remain closed.

Almost all chamber models from Percival Scientific are provided with one (1), 1 ¼” (32mm) inside diameter access port as standard equipment.

The Q10 (Additional Access Port) option provides additional ports in a Percival controlled environment and allows for the customer to specify their size and location.

This manual is intended to provide instructions on the installation, use/operation, maintenance and troubleshooting of the Q10 (Additional Access Port) option for Percival Scientific’s control environmental chambers.

## Size and Location

The diameter and location of each optional additional access port is specific to each chamber model and is typically defined by the customer during the ‘Discovery and Planning’ phase of Percival’s Proven Process.

Unless otherwise specified, each additional access port has an internal pass-through diameter of 1 ¼” (32mm).  Alternative pass-through sizes include 2”, 3” and 4” diameters, and custom sizes are also available upon request at the time of order.

# Installation

Each access port assembly is provided with one (1) pass-through sleeve (or port) and one (1) insertion plug or screw-on cap.

No tools are required for installation of port plugs or caps.

Plugs and caps can be found inside the accessories bag provided with each chamber.  Ports are left open during shipping to allow some fresh air exchange and to prevent stagnation inside the environment.

To install a port plug, insert it into its port sleeve from outside of the chamber.  The sleeve should fit snugly into its sleeve.  Fit can be adjusted by tightening (clockwise) or loosening (counterclockwise) the wingnut on the port plug.

If the access port system utilizes caps instead of plugs, place the cap over one of the threaded ends of the access port sleeve and tighten it down with a clockwise turn until finger tight.  Note: Ports with caps are most typically used on chambers with walls thicker than 2” (50.8mm).  This style of port has an internal and an external cap.

# Operation

When access ports are in use (i.e. plugs or caps removed and implements inserted), it is recommended that any remaining voids within the port are sealed to prevent ingress of ambient air into the controlled environment.  Packing foam or fabric into a void or sealing externally with tape are all effective means to minimize disturbance to the controlled environment.

When not in use, the plug (or cap) provided with each port can be installed to create an air-tight seal between the chamber and its surrounding environment.

# Maintenance

Plugs and caps should be inspected during each use for damage that could compromise their ability to hold a tight seal when installed.

Ports/caps and sleeves should be cleaned on the same schedule and in the same manner as described in the maintenance section of the chamber model’s operation manual.

# Troubleshooting

## Fit

If an access port plug does not fit properly into its port sleeve, adjust the plug’s diameter by turning its wingnut.  A clockwise adjustment will increase the port’s diameter, providing a tighter fit, and a counterclockwise adjustment will decrease the port’s diameter, providing a looser fit.

## Condensation

Access ports are generally less insulated than the chamber walls into which they are installed and are thus prone to the formation of condensation on their internal or external surfaces.  The degree to which condensation may form is dependent upon conditions inside the controlled environment relative to ambient.

When opened (plug or caps removed), each access port allows some unmanaged air exchange between the controlled environment and ambient environment which can further the development of condensation.  Generally, this condensation is minor and can be resolved by the addition of insulative material inside the port sleeve and/or measures to seal off any air exchange.

## Parts and Service

Replacement port plugs and caps can be ordered from Percival Scientific at:

Ph: 1-800-695-2743

Email: parts@percival-scientific.com

For additional questions regarding this product, please contact Percival’s customer support team at:

Phone: 1-800-695-2743

Email: customerservice@percival-scientific.com