Q5 – Additional Steel Wire Shelf

Q6 – Stainless Steel Shelving

Operator’s Manual

505 Research Drive  Perry, Iowa 50220

Toll-free 800.695.2743  515.465.9363  fax 515.465.9464

www.percival-scientific.com

[This page intentionally left blank]

# Introduction

To help customers create better science, Percival Scientific continuously studies and refines the design of its products in order to provide the best possible solutions and equipment in the industry.  As such, changes in the design and/or operation of equipment may occur without publication.  Please feel free to contact Percival Scientific directly for product updates.

Information contained within this supplemental user manual shall supersede any conflicting information contained within the basic chamber user manual.

Please read this manual carefully before proceeding with operation

# Revision Log

| Version | Date | Description |
| --- | --- | --- |
| Rev A | 7/10/2024 | Initial Release |

Introduction	3

Revision Log	4

Warranty Information	6

Definitions	8

General	8

Q5 (Additional Steel Wire Shelf)	8

Q6 (Stainless Steel Shelf)	8

Load Capacity	8

Location	9

Installation	9

Operation	9

Maintenance	10

Troubleshooting	10

Fit	10

Parts and Service	10

# Warranty Information

Equipment Limited Warranty

Thank you for purchasing equipment from Percival Scientific, Inc.  Our goal is to provide every customer with an exceptional ownership experience.  We are committed to providing products and services of the highest quality, and your satisfaction is our top priority.  In support of this objective, we present the following warranty.

Warranty Terms

Percival Scientific, Inc. (hereinafter referred to as Percival) warrants that equipment manufactured by Percival shall be free from defect in material or workmanship, when installed, operated and maintained in accordance with instructions provided by Percival.

Warranty Coverage

- Coverage period shall be twenty-four (24) months from the date of delivery

- Percival’s obligation under this warranty is limited to the repair or replacement of defect at no charge for parts, shipping and labor

- Percival reserves the right to inspect the equipment after a warranty claim is made in order to determine the proper course of corrective action

Exclusions

- Parts or elements damaged from improper installation, including mechanical, electrical and plumbing services, IT infrastructure, inadequate room ventilation, improper water quality, and ambient conditions that are out of specification

- Parts or elements damaged from improper operation

- Parts or elements damaged from improper maintenance

- Parts or elements considered to be consumable, that outlive their normal expected terms of usefulness, such as: light bulbs, filters, desiccant material and CO2 scrubbing media

- Damage resulting from shipment to or relocation within the customer’s facility after equipment has been signed for and ownership is transferred to the customer

- Modification to equipment by the customer without Percival’s consent may void this warranty

- Repair work undertaken without prior consent of Percival may void this warranty

- Damage caused by accident, fire, Force Majeure or Acts of God

Additional Terms

- This warranty is in lieu of any other warranties, expressed or implied, including merchantability or fitness for a particular purpose

- Warranty terms described herein cannot be changed without the expressed, written consent from Percival

- The owner agrees that Percival’s sole liability with respect to defective parts or materials shall be set forth in this warranty, and any claims for incidental or consequential damages are expressly excluded

- This warranty is non-transferable and applies only to the original purchaser

- Repaired or replaced parts shall be warranted only for the unexpired portion of the original warranty, unless otherwise defined

- Parts replaced under warranty shall become the property of Percival

To Make a Warranty Claim

- Document the issue with detailed notes and photographs along with the equipment’s serial number

- Contact Percival’s customer support team at customerservice@percival-scientific.com, parts@percival-scientific.com or 1-800-695-2743

- Once your claim is received, our customer support team will review it and contact you with further instructions

# Definitions

# General

Most Percival Scientific environmental chambers with adjustable shelving systems can accommodate additional shelving, beyond the standard equipment.  Shelving kits can consist of steel wire, stainless-steel wire, or solid stainless-steel construction.  These shelving systems are available as optional kits, and can be specified in quantities greater than one, if multiple units are required.

Extra shelving - additional to what is typically supplied as standard equipment with a given chamber - is available for almost all Percival controlled environmental chamber models.

## Q5 (Additional Steel Wire Shelf)

The Q5 (Additional Steel Wire Shelf) option provides one (1) white epoxy painted steel wire shelf, sized to match the base chamber’s standard shelving equipment and necessary mounting hardware.

## Q6 (Stainless Steel Shelf)

Stainless steel shelving can be added to a Percival environmental chamber as either a replacement to the standard steel shelving, or as additional hardware.  Stainless steel shelves can also be provided in wire or solid-panel construction, depending upon a chamber’s specifications and process air flow patterns.

The Q6 (Stainless Steel Shelf) option provides one (1) stainless steel shelf, sized to match the size, construction type and installation method of the base chamber’s standard shelving equipment and necessary mounting hardware.  Please refer to the chamber purchase agreement for specifics on this option, if equipped.

## Load Capacity

The static load rating of each shelf is the same as specified in the chamber model’s base manual.  It should be noted that each chamber has a total load rating, which is often less than the sum of the shelves’ individual load ratings. This requires consideration since the average allowable weight per shelf may be reduced by the addition of more shelves.

## Location

A shelf can be positioned at the operator’s discretion within the controlled environment, in the same manner as a standard shelf.

# Installation

Each optional shelf kit must be installed and supported in the same method as the other shelving, unless otherwise specified. All necessary mounting hardware will be included with each kit.

Please refer to the chamber model’s user manual for installation instructions.

A shelf and its hardware can be found inside the chamber, included with the other shelves, chamber accessories, and uninstalled parts.

No tools are typically required for installation of this component.

Be sure to position the shelf’s supports at the same level to ensure the shelf is level and its load is evenly distributed across each support point.

# Operation

Shelves serve the primary purpose of supporting experiments within a controlled environment.  In some circumstances, shelves are also used to support other equipment such as light fixtures, data acquisition equipment and sensors.

Please be aware that adding one or more shelves to a controlled environment can disturb the system’s standard air flow patterns, causing variations in the control and uniformity of process variables such as temperature, lighting and relative humidity.

# Maintenance

Follow all maintenance procedures as described in the chamber user manual as it pertains to shelves.

# Troubleshooting

## Fit

Typically, every shelf has four (4) support points and is shaped in such a way that only one orientation allows for proper engagement between the shelf and all supports. Each shelf should sit level upon its means of support point and be loaded in such a way as to establish uniform weight distribution.

## Parts and Service

Replacement parts can be ordered from Percival Scientific at:

Ph: 1-800-695-2743

Email: parts@percival-scientific.com

For additional questions regarding this product, please contact Percival’s customer support team at:

Phone: 1-800-695-2743

Email: customerservice@percival-scientific.com

Please have your chamber serial number available for faster and more efficient customer service.